硬件代码。

硬件：超声波及报警装置。

// defines pins numbers

const int trigPin = 9;

const int echoPin = 10;

float cm;

// defines variables

long duration;

int distance;

void setup()

{

Serial.begin(9600);

pinMode(11, OUTPUT);  //发红色光

pinMode(12, OUTPUT);  //发蓝色光

pinMode(13, OUTPUT);   //发绿色光

pinMode(trigPin, OUTPUT);   // Sets the trigPin as an Output

pinMode(6, OUTPUT);   //VIBRATOR

pinMode(3,OUTPUT);  //BUZZER

pinMode(echoPin, INPUT);  // Sets the echoPin as an Input

}

void loop()

{

//发一个10ms的高脉冲去触发TrigPin

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

cm = pulseIn(echoPin, HIGH) / 58.0; //算成厘米

cm = (int(cm * 100.0)) / 100.0; //保留两位小数

Serial.print(cm);

Serial.print("cm");

Serial.println();

delay(1000);

// Reads the echoPin, returns the sound wave travel time in microseconds

duration = pulseIn(echoPin, HIGH);

// Calculating the distance

distance= duration*0.034/2;

if(cm<25)

{

Serial.print("warning");

digitalWrite(11,HIGH); digitalWrite(12,LOW); digitalWrite(13,LOW);

analogWrite(6,140);

tone(3,1000,1000);

}

else if(cm<50)

{

Serial.print("narrow");

digitalWrite(11,LOW); digitalWrite(12,HIGH); digitalWrite(13,LOW);

analogWrite(6,90);

tone(3,1000,1000);

}

else if(cm<75)

{

Serial.print("OK");

digitalWrite(11,LOW); digitalWrite(12,LOW); digitalWrite(13,HIGH);

analogWrite(6,60);

analogWrite(3,255);

}

else if(cm<100)

{

Serial.print("safe");

digitalWrite(11,HIGH); digitalWrite(12,LOW); digitalWrite(13,LOW);

analogWrite(6,30);

analogWrite(3,255);

}

else

{

Serial.print("very safe");

digitalWrite(11,LOW); digitalWrite(12,LOW); digitalWrite(13,HIGH);

analogWrite(6,0);

analogWrite(3,0);

noTone(3);

}

}

GPS 的发送

#include<Servo.h>

#include <TimerOne.h>

#define GPSget Serial2

#define DebugSerial Serial

#define GprsSerail Serial3

#define GPRMC_TERM "$GPRMC,"    //定义要解析的指令，因为这条指令包含定位和时间信息

#define Success 1U

#define Failure 0U

int L=28;

unsigned long  Time_Cont = 0;       //定时器计数器

char a_y[]="1";

char a_n[]="0";

const unsigned int gprsRxBufferLength = 600;

char gprsRxBuffer[gprsRxBufferLength];

unsigned int gprsBufferCount = 0;

char OneNetServer[] = "api.heclouds.com";       //不需要修改

char device_id[] = "542005309";    //修改为自己的设备ID

char API_KEY[] = "IvYhykyRqpoN6zhm5Wz1HKy=NJg=";    //修改为自己的API_KEY

char sensor_gps[] = "loc";

char lon_str[15];

char lat_str[15];

char nmeaSentence[68];

String latitude;    //纬度

String longitude;   //经度

int isfull=0;

Servo myServo;

int buzzer=2;

int red_line=6;

int TrigPin3=7;

int EchoPin3=8;

int ledPin=12;

int button=13;

int distance3;

int Switch;

void setup() {

pinMode(EchoPin3,INPUT);

pinMode(TrigPin3,OUTPUT);

pinMode(buzzer,OUTPUT);

pinMode(ledPin,OUTPUT);

pinMode(red_line,INPUT);

pinMode(button,INPUT_PULLUP);

myServo.attach(9);

myServo.write(90);

DebugSerial.begin(19200);

GprsSerail.begin(19200);

GPSget.begin(9600);

Timer1.initialize(1000);

Timer1.attachInterrupt(Timer1_handler);

initGprs();

DebugSerial.println("\r\nsetup end!");

}

void loop() {

distance3=ping3();

if(distance3<=5&&distance3>2)

isfull=isfull+1;

int buttonState=digitalRead(button);

int red_line_get=digitalRead(red_line);

if(buttonState==1)

{

myServo.write(21);/*垃圾桶打开*/

Switch=1;

isfull=-1300;

}

else

{

if((isfull>270)&&(Switch==0))

{

delay(1000);

//initGprs();

while(buttonState==0)

{

for(int k=0;k<10;k++)

alarm();

for(int pl=0;pl<2;pl++)

gpsToOnenet(1);

buttonState=digitalRead(button);

}

}

else

{

if(red_line_get==0)

{

myServo.write(21);

delay(3200);

isfull=0;

for(int i=21;i<=86;i++)

{

myServo.write(i);

delay(55);

}

gpsToOnenet(0);

}

else

{

if(Switch==1)

{

delay(1000);

for(int i=21;i<=86;i++)

{

myServo.write(i);

delay(55);

}

isfull=0;

Switch=0;

}

}

}

}

}

int ping3()

{

int duration3;

digitalWrite(TrigPin3,LOW);

delayMicroseconds(2);

digitalWrite(TrigPin3,HIGH);

delayMicroseconds(10);

digitalWrite(TrigPin3,LOW);

duration3=pulseIn(EchoPin3,HIGH)/58;

//Serial.println(duration3);

return duration3;

}

void alarm()

{

digitalWrite(buzzer,HIGH);

digitalWrite(ledPin,HIGH);

delay(100);

digitalWrite(buzzer,LOW);

digitalWrite(ledPin,LOW);

delay(100);

}

void gpsToOnenet(int is){

// For one second we parse GPS data and report some key values

for (unsigned long start = millis(); millis() - start < 1000;)  //一秒钟内不停扫描GPS信息

{

while (GPSget.available())  //串口获取到数据开始解析

{

char c = GPSget.read(); //读取一个字节获取的数据

switch(c)         //判断该字节的值

{

case '$':         //若是$，则说明是一帧数据的开始

GPSget.readBytesUntil('*', nmeaSentence, 67);   //读取接下来的数据，存放在nmeaSentence字符数组中，最大存放67个字节

// GPSget.println(nmeaSentence);

latitude = parseGprmcLat(nmeaSentence); //获取纬度值 String

longitude = parseGprmcLon(nmeaSentence);//获取经度值  String

if((latitude > "")&&(longitude > ""))

{

for(int c=0;c<=9;c++)

lon_str[c]=longitude.charAt(c);

for(int b=0;b<=8;b++)

lat_str[b]=latitude.charAt(b);

DebugSerial.println(lat_str);

DebugSerial.println(lon_str);

postGpsDataToOneNet(API_KEY,device_id,sensor_gps,lon_str,lat_str,is);  //开始传输数据

}

}

}

}

delay(2000);

delay(5000);

}

double longitudeToOnenetFormat(char* lon_str_temp)

{

double lon_temp = 0;

long lon_Onenet = 0;

int dd_int = 0;

long mm_int = 0;

double lon_Onenet_double = 0;

lon_temp = atof(lon_str_temp);

lon_Onenet =lon_temp*100000;  //转换为整数

dd_int = lon_Onenet/10000000; //取出dd

mm_int = lon_Onenet%10000000;  //取出MM部分

lon_Onenet_double = dd_int + (double)mm_int/60/100000;//换算为Onenet格式

return lon_Onenet_double;

}

double latitudeToOnenetFormat(char* lat_str_temp)

{

double lat_temp = 0;

long lat_Onenet = 0;

int dd_int = 0;

long mm_int = 0;

double lat_Onenet_double = 0;

lat_temp = atof(lat_str_temp);

lat_Onenet =lat_temp*100000;  //转换为整数

dd_int = lat_Onenet/10000000; //取出dd

mm_int = lat_Onenet%10000000;  //取出MM部分

lat_Onenet_double = dd_int + (double)mm_int/60/100000;//换算为Onenet格式

return lat_Onenet_double;

}

void postGpsDataToOneNet(char* API_VALUE_temp,char* device_id_temp,char* sensor_id_temp,char* lon_temp,char* lat_temp,int a)

{

char send_buf[400]= {0};

char text[100] = {0};

char tmp[25] = {0};

char b[]="0";

char lon_str_end[15] = {0};

char lat_str_end[15] = {0};

dtostrf(longitudeToOnenetFormat(lon_temp),3,6, lon_str_end);  //转换成字符串输出

dtostrf(latitudeToOnenetFormat(lat_temp),2,6, lat_str_end);  //转换成字符串输出

//连接服务器

memset(send_buf, 0, 400);    //清空

strcpy(send_buf, "AT+CIPSTART=\"TCP\",\"");

strcat(send_buf, OneNetServer);

strcat(send_buf, "\",80\r\n");

if (sendCommand(send_buf, "CONNECT", 10000, 5) == Success);

else GPRS_ERROR(7);

//发送数据

if (sendCommand("AT+CIPSEND\r\n", ">", 3000, 1) == Success);

else GPRS_ERROR(8);

memset(send_buf, 0, 400);    //清空

/*准备JSON串*/

//ARDUINO平台不支持sprintf的double的打印，只能转换到字符串然后打印

if(a==0)

sprintf(text,"{\"datastreams\":[{\"id\":\"%s\",\"datapoints\":[{\"value\":{\"lon\":%s,\"lat\":%s,\"is\":%s}}]}]}",sensor_id_temp,lon_str_end,lat_str_end,b);

else

sprintf(text,"{\"datastreams\":[{\"id\":\"%s\",\"datapoints\":[{\"value\":{\"lon\":%s,\"lat\":%s}}]}]}",sensor_id_temp,lon_str_end,lat_str_end);

/*准备HTTP报头*/

send_buf[0] = 0;

strcat(send_buf,"POST /devices/");

strcat(send_buf,device_id_temp);

strcat(send_buf,"/datapoints HTTP/1.1\r\n");//注意后面必须加上\r\n

strcat(send_buf,"api-key:");

strcat(send_buf,API_VALUE_temp);

strcat(send_buf,"\r\n");

strcat(send_buf,"Host:");

strcat(send_buf,OneNetServer);

strcat(send_buf,"\r\n");

sprintf(tmp,"Content-Length:%d\r\n\r\n", strlen(text));//计算JSON串长度

strcat(send_buf,tmp);

strcat(send_buf,text);

if (sendCommand(send_buf, send_buf, 3000, 1) == Success);

else GPRS_ERROR(9);

char sendCom[2] = {0x1A};

if (sendCommand(sendCom, "\"succ\"}", 3000, 1) == Success);

else GPRS_ERROR(10);

if (sendCommand("AT+CIPCLOSE\r\n", "CLOSE OK", 3000, 1) == Success);

else GPRS_ERROR(11);

if (sendCommand("AT+CIPSHUT\r\n", "SHUT OK", 3000, 1) == Success);

else GPRS_ERROR(11);

}

void initGprs()

{

if (sendCommand("AT\r\n", "OK", 3000, 10) == Success);

else GPRS_ERROR(1);

if (sendCommand("AT+CREG?\r\n", "OK", 3000, 10) == Success);

else GPRS_ERROR(2);

if (sendCommand("AT+CGCLASS=\"B\"\r\n", "OK", 3000, 2) == Success);

else GPRS_ERROR(3);

if (sendCommand("AT+CGDCONT=1,\"IP\",\"CMNET\"\r\n", "OK", 3000, 2) == Success);

else GPRS_ERROR(4);

if (sendCommand("AT+CGATT=1\r\n", "OK", 3000, 2) == Success);

else GPRS_ERROR(5);

if (sendCommand("AT+CLPORT=\"TCP\",\"2000\"\r\n", "OK", 3000, 2) == Success);

else GPRS_ERROR(6);

}

void(* resetFunc) (void) = 0; //制造重启命令

void GPRS_ERROR(int num)

{

DebugSerial.print("ERROR");

DebugSerial.println(num);

while (1)

{

digitalWrite(L, HIGH);

delay(300);

digitalWrite(L, LOW);

delay(300);

if (sendCommand("AT\r\n", "OK", 100, 10) == Success)

{

DebugSerial.print("\r\nRESET!!!!!!\r\n");

resetFunc();

}

}

}

unsigned int sendCommand(char *Command, char *Response, unsigned long Timeout, unsigned char Retry)

{

clrGprsRxBuffer();

for (unsigned char n = 0; n < Retry; n++)

{

DebugSerial.print("\r\n---------send AT Command:---------\r\n");

DebugSerial.write(Command);

GprsSerail.write(Command);

Time_Cont = 0;

while (Time_Cont < Timeout)

{

gprsReadBuffer();

if(strstr(gprsRxBuffer, Response) != NULL)

{

DebugSerial.print("\r\n==========receive AT Command:==========\r\n");

DebugSerial.print(gprsRxBuffer); //输出接收到的信息

clrGprsRxBuffer();

return Success;

}

}

Time_Cont = 0;

}

DebugSerial.print("\r\n==========receive AT Command:==========\r\n");

DebugSerial.print(gprsRxBuffer);//输出接收到的信息

clrGprsRxBuffer();

return Failure;

}

void Timer1_handler(void)

{

Time_Cont++;

}

void gprsReadBuffer() {

while (GprsSerail.available())

{

gprsRxBuffer[gprsBufferCount++] = GprsSerail.read();

if (gprsBufferCount == gprsRxBufferLength)clrGprsRxBuffer();

}

}

void clrGprsRxBuffer(void)

{

memset(gprsRxBuffer, 0, gprsRxBufferLength);      //清空

gprsBufferCount = 0;

}

String parseGprmcLat(String s)

{

int pLoc = 0; //paramater location pointer

int lEndLoc = 0; //lat parameter end location

String lat;

if(s.substring(0,4) == "GPRM")

{

//Serial.println(s);

for(int i = 0; i < 5; i++)

{

if(i < 3)

{

pLoc = s.indexOf(',', pLoc+1);

}

if(i == 3)

{

lEndLoc = s.indexOf(',', pLoc+1);

lat = s.substring(pLoc+1, lEndLoc);

}

}

return lat;

}

}

String parseGprmcLon(String s)

{

int pLoc = 0; //paramater location pointer

int lEndLoc = 0; //lat parameter end location

String lon;

if(s.substring(0,4) == "GPRM")

{

//Serial.println(s);

for(int i = 0; i < 7; i++)

{

if(i < 5)

{

pLoc = s.indexOf(',', pLoc+1);

}

if(i == 5)

{

lEndLoc = s.indexOf(',', pLoc+1);

lon = s.substring(pLoc+1, lEndLoc);

}

}

return lon;

}

}

Onenet:

微信小程序：（核心代码及其示例）

<<map id="myMap" style="width: {{mapWidth}}rpx; height: {{mapHeight}}rpx;" latitude="{{lonn1}}" longitude="{{latn1}}" 	arkers="{{markers}}" show-location bindmarkertap="selectMarket" 	include-points="{{markers}}" bindmarkertap="toaddress"></map>

- 小程序获取服务器数据部分：（js)

- onLoad: function () {

- var that = this//不要漏了这句，很重要

- wx.request({

- url: 'http://api.heclouds.com/devices/576932530',

- headers: {

- 'Content-Type': 'application/json',

- 'api-key':'zoRJuv0JCc5c5Is8U2DlD62nDRo='

- },

- success: function (res) {

- that.setData({

- longitude: res.data.longitude,

- latitude: res.data.latitude,

- //res代表success函数的事件对，data是固定的，fengxiang是是上面json数据中fengxiang

- })

- }

- })

- },

- Api

- <view class="" hover-class="none" hover-stop-propagation="false" style="width: 100%; height: 100%;">

- <map id="map" longitude="{{longitude}}" latitude=" {{latitude}}" markers="{{markers}}" scale="13" show-location style="width: 100%; height: 100%;"></map>

- <view></view>

- </view>

- Wxml function

- page{

- height: 96%;

- }

- Wxss function

- <view class="root">

- <image class='background-image' src='./background.jpg' mode="aspectFill"></image>

- <button class='gps' bindtap="getLocation">

- <image class='gpsimage' src='./gps1.png'></image>

- <view>{{locationButton}}</view>

- </button>

- </view>

- Wxml index

- page{

- width:100%;

- height:100%;

- overflow: hidden;

- }

- .header{

- text-align: center;

- color: rgb(0, 0, 0);

- font-size:  23px;

- padding: 23px 9%;

- line-height: 1.5;

- }

- .gps{

- position: top;

- width: 300rpx;

- height: 300rpx;

- //margin-top: 20rpx;

- color: #000000;

- border-radius: 0rpx;

- display: flex;

- flex-direction: column;

- font-size: 40rpx;

- background-color: transparent;   /*按钮透明*/

- border:none;       /*设置按钮边框*/

- // background-color: #70c7da;

- }

- .gpsimage{

- top: 100px;

- width: 300rpx;

- height: 300rpx;

- }

- .root {

- width: 100%;

- height: 100%;

- background-color: #70c7da;

- position: relative;

- }

- .background-image{

- height : 100%;

- position: absolute;

- width: 100%;

- left: 0px;

- top: 0px;

- }

- Wxss index

- Page({

- data:{

- header: "下图是您的家人在的位置",

- locationButton: "点击获取家人的位置信息"

- },

- getLocation: function () {

- wx.navigateTo({

- url: '../function/function',

- })

- }

- })

- Js index

- Page({

- data: {

- header: "欢迎来到盲人拐杖家属客户端",

- locationButton: "点击按钮进入程式中"

- },

- getLocation: function(){

- wx.navigateTo({

- url: '../index/index',

- })

- }

- })

- Start js

- <view class='header'>{{header}}</view>

- <button bindtap="getLocation" class="locationButton">{{locationButton}}</button>

- Start wxml

- page{

- background:#80acdfc0;

- }

- .header{

- text-align: center;

- color: darkgreen;

- font-size:  40px;

- padding: 23px 9%;

- line-height: 1.5;

- }

- .locationButton{

- size: 200px;

- background-color: rgb(106, 143, 150);

- color: #3813db;

- font-size: 25px;

- top : 45px;

- }

- Wxss start

- {

- "pages": [

- "pages/start/start",

- "pages/index/index",

- "pages/function/function"

- ],

- "window": {

- "backgroundTextStyle": "light",

- "navigationBarBackgroundColor": "#fff",

- "navigationBarTitleText": "盲人拐杖",

- "navigationBarTextStyle": "black"

- },

- "style": "v2",

- "sitemapLocation": "sitemap.json"

- }

- App jason

展示图片：